Compound Solvent Handling, Feasibility, Plate Sealing, and Storage SOP

Compound Management Assistant MVP | SOP-Driven Handling, Feasibility, and Plate Lifecycle Rules

| SOP ID | CM-SOLV-PLT-001 | Version | 1.1 |
| --- | --- | --- | --- |
| Effective Date | 2026-07-23 | Owner | Compound Management Team |
| Applies To | DMSO, Methanol, Ethanol, Acetonitrile, Water, PBS, Buffer; source, intermediate, and assay plates | System Use | CompoundOps Assistant |
| Document Type | Simplified Training SOP | Regulatory Status | Educational mock SOP; not for regulated lab use |

Executive Use Statement This SOP provides deterministic rules for solvent handling, plate sealing, heat-seal eligibility, cold or frozen storage, incubation review, transport, inventory feasibility, freeze-thaw limits, expiration, and residual-volume calculations. The CompoundOps Assistant should generate rule-based verdicts before providing any AI explanation.

# 1. Purpose

This SOP defines standardized handling, storage, feasibility assessment, dispensing, sealing, heat exposure, incubation, and transport requirements for compound stocks and compound-containing plates. It is designed to support the CompoundOps Assistant and provide consistent guidance for scientists and compound management users during assay planning and plate preparation.

# 2. Scope

This SOP applies to compound inventory records, source plates, intermediate plates, assay-ready plates, acoustic dispensing workflows, manual plate preparation, and plate storage workflows involving the following solvent systems:

DMSO

Methanol

Ethanol

Acetonitrile

Water

PBS

Aqueous or custom buffer systems

# 3. Universal Feasibility Requirements

| Rule ID | Requirement | System Decision | Evidence Output |
| --- | --- | --- | --- |
| GEN-001 | Compound must exist in inventory and have a traceable storage location. | INSUFFICIENT INFORMATION if missing. | Compound ID, location |
| GEN-002 | Inventory status must be Available. | PASS if Available; BLOCK if unavailable, expired, low-volume, quarantined, in transit, or depleted. | Inventory status |
| GEN-003 | Compound must not be expired on request date. | PASS when expiration is valid; BLOCK when expired. | Expiration date, request date |
| GEN-004 | Freeze-thaw count must be within allowed limit. | PASS at 0-4 cycles; BLOCK at 5 or more; insufficient if missing. | Freeze-thaw count, rule limit |
| GEN-005 | Required volume must not exceed usable volume after residual volume is reserved. | PASS if required <= usable; BLOCK if required > usable. | Available, residual, usable, required volume |
| GEN-006 | Container or plate must be visually acceptable if inspection information is recorded. | BLOCK if precipitation, phase separation, leakage, failed seal, or compromise is recorded. | Inspection status or handling note |

# 4. Universal Decision Categories

| Decision | Definition |
| --- | --- |
| PASS | The check meets all requirements. |
| WARNING | The material or plate may be usable, but review or caution is recommended. |
| BLOCK | The material or plate should not be dispensed, stored, shipped, or used under the current conditions. |
| INSUFFICIENT INFORMATION | A required field is missing or invalid; the app should not assume feasibility. |
| FEASIBLE | All blocking checks pass. |
| FEASIBLE WITH WARNING | No blocking rule failed, but at least one warning check requires review. |
| NOT FEASIBLE | At least one blocking rule failed. |

# 5. DMSO Handling SOP

DMSO is commonly used for small-molecule stocks and acoustic dispensing workflows because many compounds are soluble in DMSO and the solvent can be normalized by backfill or vehicle controls.

## Business Rules

| Rule ID | Requirement | System Decision | Evidence Output |
| --- | --- | --- | --- |
| DMSO-001 | Final assay DMSO must be <= 0.5% v/v. | PASS at or below 0.5%; BLOCK above limit unless approved exception exists. | Calculated DMSO %, limit, result |
| DMSO-002 | Intermediate dilution DMSO should remain <= 10% v/v. | PASS at or below 10%; WARNING above 10% and require reformulation or exception review. | Intermediate DMSO %, limit, warning |
| DMSO-003 | Vehicle controls should match the highest DMSO level used in test wells. | WARNING when matched vehicle control is missing. | Control wells, highest DMSO % |
| DMSO-004 | DMSO stocks should be visually inspected before dispensing. | BLOCK if precipitation is recorded. | Inspection status / handling note |
| DMSO-005 | DMSO stocks should minimize water exposure and remain closed when not actively handled. | WARNING when prolonged exposure is recorded. | Handling note |
| DMSO-006 | DMSO stocks should generally be stored frozen unless compound-specific guidance exists. | WARNING or review when DMSO stock is not frozen. | Solvent, storage temperature |

## Handling Guidance

Confirm final solvent percentage during assay design.

Use vehicle controls matching the highest solvent exposure.

Inspect for precipitation or phase separation before dispensing.

Keep containers closed when not actively aspirating or dispensing.

# 6. Methanol Handling SOP

Methanol may be used for compounds requiring an alcohol-based solvent system. Methanol is volatile, flammable, and toxic by inhalation, ingestion, or skin exposure, so containment and compatibility review are important.

## Business Rules

| Rule ID | Requirement | System Decision | Evidence Output |
| --- | --- | --- | --- |
| MET-001 | Methanol-containing stocks require assay compatibility review. | WARNING if solvent tolerance is unknown. | Solvent, assay compatibility field |
| MET-002 | Methanol containers must remain tightly sealed. | WARNING if prolonged open exposure is recorded. | Handling note |
| MET-003 | Methanol stocks must remain visually clear before dispensing. | BLOCK if precipitation or cloudiness is recorded. | Inspection status |
| MET-004 | Storage temperature and location must be recorded. | INSUFFICIENT INFORMATION if missing. | Storage temperature, location |
| MET-005 | High final methanol content should be flagged for review. | WARNING when final solvent percentage is not calculated or exceeds local assay tolerance. | Calculated solvent %, tolerance if available |

## Handling Guidance

Minimize open-container time.

Use appropriate containment for volatile solvents.

Confirm the assay can tolerate methanol before plate generation.

Prefer fresh transfer plates when evaporation risk is high.

# 7. Ethanol Handling SOP

Ethanol may be used for alcohol-compatible compounds or assay systems. Ethanol is volatile and flammable, and final assay compatibility should be reviewed before use.

## Business Rules

| Rule ID | Requirement | System Decision | Evidence Output |
| --- | --- | --- | --- |
| ETH-001 | Ethanol-containing stocks require assay compatibility review. | WARNING if solvent tolerance is unknown. | Solvent, assay compatibility field |
| ETH-002 | Ethanol containers should be capped immediately after handling. | WARNING if prolonged open exposure is recorded. | Handling note |
| ETH-003 | Ethanol stocks must remain visually clear. | BLOCK if precipitation or phase separation is recorded. | Inspection status |
| ETH-004 | Final assay solvent concentration should be documented when used. | WARNING if final solvent percentage is missing. | Calculated solvent % |
| ETH-005 | Evaporation risk should be considered for small-volume transfers. | WARNING for small open volumes or prolonged handling. | Container, volume, handling note |

## Handling Guidance

Keep containers capped when not in use.

Minimize evaporation during setup.

Verify assay tolerance when ethanol is introduced into final wells.

Inspect stocks before dispensing.

# 8. Acetonitrile Handling SOP

Acetonitrile may appear in analytical or compound-preparation workflows and requires compatibility and safe-handling review due to volatility, flammability, and toxicity concerns.

## Business Rules

| Rule ID | Requirement | System Decision | Evidence Output |
| --- | --- | --- | --- |
| ACN-001 | Acetonitrile-containing stocks require assay compatibility review. | WARNING if compatibility is unknown. | Solvent, assay compatibility field |
| ACN-002 | Containers must remain closed except during active handling. | WARNING if prolonged exposure is recorded. | Handling note |
| ACN-003 | Stock must remain visually clear. | BLOCK if precipitation or cloudiness is recorded. | Inspection status |
| ACN-004 | Storage location must be recorded. | INSUFFICIENT INFORMATION if missing. | Storage location |
| ACN-005 | Final assay acetonitrile content should be calculated or reviewed. | WARNING if final solvent concentration is missing or above assay tolerance. | Calculated solvent %, tolerance if available |

## Handling Guidance

Minimize open-container exposure.

Confirm downstream assay compatibility.

Use appropriate containment and chemical-handling controls.

Record source and destination details during transfer.

# 9. Water-Based Stock SOP

Water-based stocks are common for aqueous-compatible compounds. The primary concerns are stability, precipitation, microbial or contamination risk where applicable, and documented storage conditions.

## Business Rules

| Rule ID | Requirement | System Decision | Evidence Output |
| --- | --- | --- | --- |
| AQ-001 | Solution must remain visually clear. | BLOCK if precipitation or cloudiness is recorded. | Inspection status |
| AQ-002 | Storage temperature must be recorded. | INSUFFICIENT INFORMATION if missing. | Storage temperature |
| AQ-003 | Expiration date must be valid. | PASS when valid; BLOCK if expired. | Expiration date, request date |
| AQ-004 | Concentration must be documented. | INSUFFICIENT INFORMATION if missing. | Stock concentration |
| AQ-005 | Location must be recorded and traceable. | INSUFFICIENT INFORMATION if missing. | Storage location |

## Handling Guidance

Inspect for precipitation before dispensing.

Review stability after thawing or storage.

Confirm storage temperature and expiry before use.

Avoid assuming stability when records are missing.

# 10. PBS Handling SOP

PBS is a phosphate-buffered saline system commonly used in biological workflows. PBS formulations should remain clear, appropriate for the assay system, and traceable by storage and composition records.

## Business Rules

| Rule ID | Requirement | System Decision | Evidence Output |
| --- | --- | --- | --- |
| PBS-001 | PBS stock must remain visually clear. | BLOCK if precipitation or cloudiness is recorded. | Inspection status |
| PBS-002 | Storage location must be recorded. | INSUFFICIENT INFORMATION if missing. | Location |
| PBS-003 | Expiration date must be valid. | PASS when valid; BLOCK if expired. | Expiration date, request date |
| PBS-004 | Composition or formulation should be documented when relevant. | WARNING if composition is unknown. | Buffer formulation field or note |
| PBS-005 | pH or compatibility review should be performed when assay requires it. | WARNING if assay compatibility is unknown. | Assay compatibility note |

## Handling Guidance

Inspect for cloudiness, precipitation, or crystallization.

Confirm storage condition and expiration date.

Confirm formulation compatibility with assay conditions.

Document any pH-sensitive requirements.

# 11. Buffer-Based Stock SOP

Buffer-based stocks include custom aqueous formulations. The primary considerations are documented buffer composition, storage condition, concentration, location, and compatibility with downstream assay components.

## Business Rules

| Rule ID | Requirement | System Decision | Evidence Output |
| --- | --- | --- | --- |
| BUF-001 | Buffer formulation must be documented when required. | WARNING or INSUFFICIENT INFORMATION if missing, depending on assay requirement. | Buffer composition field or note |
| BUF-002 | Storage temperature must be recorded. | INSUFFICIENT INFORMATION if missing. | Storage temperature |
| BUF-003 | No visible precipitation or phase separation. | BLOCK if compromised. | Inspection status |
| BUF-004 | Stock concentration must be documented. | INSUFFICIENT INFORMATION if missing. | Stock concentration |
| BUF-005 | Location must be recorded. | INSUFFICIENT INFORMATION if missing. | Storage location |

## Handling Guidance

Verify formulation before dilution planning.

Confirm compatibility with assay buffer and biology.

Inspect before dispensing.

Treat missing buffer composition as a review item.

# 12. Plate Sealing Requirements

All compound-containing plates should be sealed promptly after preparation to reduce contamination risk, evaporation, solvent loss, and handling variability. The appropriate seal depends on plate type, solvent system, downstream workflow, and whether the plate is a source, intermediate, or assay-ready plate.

| Rule ID | Requirement | System Decision | Evidence Output |
| --- | --- | --- | --- |
| PS-001 | All compound-containing source, intermediate, and assay-ready plates must be sealed after dispensing. | PASS when documented seal is applied; WARNING if seal type is missing; BLOCK if plate is intentionally left unsealed without justification. | Plate ID, seal type, timestamp |
| PS-002 | Seal must be compatible with the solvent system present in the plate. | PASS when compatible; WARNING if compatibility is unknown; BLOCK if known incompatible seal is used. | Solvent, seal material, compatibility note |
| PS-003 | Source and intermediate plates containing volatile solvents require evaporation-resistant seals. | WARNING if volatile solvent plate uses undocumented or non-resistant seal. | Solvent, seal type, handling note |
| PS-004 | Assay plates requiring optical readout should use assay-compatible optical seals when applicable. | WARNING if seal type may interfere with readout. | Assay type, readout mode, seal type |
| PS-005 | Any evidence of failed seal, leakage, condensation compromise, or plate deformation must be reviewed. | BLOCK if seal failure compromises sample integrity. | Seal inspection status |

## Recommended Seal Type by Plate Category

| Plate Category | Common Seal Type | When Appropriate | App Decision Logic |
| --- | --- | --- | --- |
| Source Plate | Pierceable foil, chemical-resistant foil, or heat seal | Longer handling, acoustic source workflows, volatile or organic solvent stocks | Require documented compatibility and seal type |
| Intermediate Plate | Foil or solvent-resistant heat seal | Prepared dilution plate, short-term source for assay transfer | Flag if storage duration or seal compatibility is unknown |
| Assay-Ready Plate | Optically clear film, adhesive film, foil seal as assay-compatible | Biological assay, incubation, readout, or transport | Require assay/readout compatibility if optical measurement is planned |

# 13. Heat Sealing and Heat Exposure Requirements

Heat sealing may be appropriate for certain plate and seal combinations, but the app should not assume heat-seal compatibility. Heat exposure should be reviewed for compound stability, solvent volatility, plate material, and assay impact.

| Rule ID | Requirement | System Decision | Evidence Output |
| --- | --- | --- | --- |
| HT-001 | Plate type must be documented as compatible with heat sealing before heat seal is selected. | PASS if compatible; BLOCK if incompatible; WARNING if unknown. | Plate type, manufacturer or compatibility note |
| HT-002 | Seal material must be compatible with the solvent system and heat-seal process. | PASS if compatible; WARNING if compatibility unknown. | Solvent, seal type, material note |
| HT-003 | Compound-only source or intermediate plates should not be heat-treated for stability purposes unless documented stability evidence exists. | WARNING or BLOCK when heat treatment is requested without stability support. | Treatment request, stability evidence |
| HT-004 | Assay plates may only be incubated or heat-exposed according to assay-defined conditions. | PASS when assay temperature and duration are documented; WARNING if incomplete. | Temperature, duration, assay type |
| HT-005 | Any plate deformation, condensation, evaporation, or seal distortion after heat exposure requires review. | BLOCK if sample integrity is compromised. | Post-seal or post-incubation inspection |

Important Implementation Note The CompoundOps Assistant should ask for plate type, seal type, and intended storage or incubation condition before allowing heat sealing, incubation, or heat exposure to be marked as PASS. If these values are missing, the app should return WARNING or INSUFFICIENT INFORMATION rather than assuming compatibility.

# 14. Plate Storage Requirements

Plate storage conditions should be selected based on solvent, compound stability, plate type, and whether the plate is a long-term source plate, short-term intermediate plate, or assay-ready plate.

| Rule ID | Requirement | System Decision | Evidence Output |
| --- | --- | --- | --- |
| STP-001 | DMSO source plates should generally be stored frozen unless compound-specific guidance states otherwise. | PASS when frozen storage is documented; WARNING for non-frozen storage without justification. | Solvent, storage temperature, exception note |
| STP-002 | Aqueous, PBS, and buffer plates should be stored under documented cold or assay-defined conditions. | PASS when storage condition is documented; WARNING if unspecified. | Solvent, storage temperature |
| STP-003 | Intermediate plates should be used within validated or documented hold-time limits. | WARNING if hold time is missing or exceeds allowed window. | Preparation time, planned use time |
| STP-004 | Assay-ready plates should follow assay-specific storage or incubation instructions. | INSUFFICIENT INFORMATION if storage/incubation condition is missing. | Assay protocol, temperature, duration |
| STP-005 | Frozen storage or thawing events must be considered in freeze-thaw tracking. | UPDATE or FLAG when freeze-thaw event occurs. | Previous count, new count, plate or container ID |

| Plate Type | Typical Storage Decision | Risk to Review | Assistant Recommendation |
| --- | --- | --- | --- |
| Source Plate | Frozen or source-condition storage based on solvent and inventory guidance | Freeze-thaw impact, evaporation, seal integrity | Track storage condition, seal, location, and freeze-thaw event |
| Intermediate Plate | Short-term controlled storage based on solvent and dilution stability | Evaporation, concentration drift, microbial/precipitation risk for aqueous systems | Recommend use within documented hold time |
| Assay-Ready Plate | Assay-specific storage or incubation condition | Cell/biology compatibility, readout interference, edge effects, seal compatibility | Require assay temperature, duration, and readout compatibility before PASS |

# 15. Incubation Requirements

Incubation rules differ for compound-only plates and biological assay plates. Compound-only source and intermediate plates should generally not be incubated unless there is a documented purpose and stability justification. Assay-ready plates may be incubated when the assay protocol defines temperature and duration.

| Rule ID | Requirement | System Decision | Evidence Output |
| --- | --- | --- | --- |
| INC-001 | Compound-only source plates are not intended for biological incubation. | WARNING when incubation is requested; BLOCK if stability risk is known. | Plate type, requested temperature, duration |
| INC-002 | Intermediate plates require documented stability or hold-time guidance before incubation. | WARNING if incubation condition is not documented. | Intermediate ID, temperature, duration |
| INC-003 | Assay-ready plates may be incubated according to assay-defined conditions. | PASS when temperature and duration are recorded. | Assay type, temperature, duration |
| INC-004 | Elevated temperature exposure requires review when compound stability is unknown. | WARNING or BLOCK depending on stability evidence. | Temperature, duration, stability note |
| INC-005 | Seal compatibility must be confirmed for incubated plates. | WARNING if seal compatibility is unknown. | Seal type, incubation condition |

# 16. Plate Transport and Shipping Requirements

Compound-containing plates that are transported between benches, rooms, buildings, or sites should remain sealed and traceable. Temperature requirements should match the plate type, solvent, and intended use.

| Rule ID | Requirement | System Decision | Evidence Output |
| --- | --- | --- | --- |
| TRN-001 | Plates must remain sealed during transport. | BLOCK if unsealed during transport. | Plate ID, seal status |
| TRN-002 | Transport temperature must match required storage condition when specified. | WARNING if temperature record is missing; BLOCK if excursion compromises material. | Transport temperature record |
| TRN-003 | Frozen plates should maintain frozen-condition handling when required by solvent or stability guidance. | WARNING if frozen transport evidence is missing. | Temperature record, packaging note |
| TRN-004 | Chain-of-custody information should be recorded for cross-site shipment. | INSUFFICIENT INFORMATION if sender, receiver, date, or plate ID is missing. | Sender, receiver, date, plate ID |
| TRN-005 | Seal inspection should be performed after transport. | WARNING or BLOCK if seal damage is observed. | Post-transport inspection |

# 17. Freeze-Thaw Requirements for All Solvents

| Rule ID | Requirement | System Decision | Evidence Output |
| --- | --- | --- | --- |
| FT-001 | Maximum permitted freeze-thaw count is <= 4 cycles. | PASS at 0-4 cycles; BLOCK at 5 or more cycles. | Recorded count, limit, result |
| FT-002 | Freeze-thaw count is required. | INSUFFICIENT INFORMATION when count is blank or unknown. | Container and cycle record |
| FT-003 | Do not combine freeze-thaw counts across separate aliquots or plates. | FLAG duplicate or missing container identifiers. | Compound ID and container ID |
| FT-004 | Increment count after a thawed container or qualifying plate is returned to frozen storage. | UPDATE inventory after completed handling. | Previous and new count |

# 18. Residual Volume and Volume Feasibility Rules

| Rule ID | Requirement | System Decision | Evidence Output |
| --- | --- | --- | --- |
| VOL-001 | A minimum residual volume of 10 uL must remain after dispensing. | BLOCK if required volume exceeds available volume minus 10 uL. | Available, residual, usable, required |
| VOL-002 | Echo or source dead volume must be included when calculating source requirement. | PASS if total required plus dead volume is within usable volume. | Transfer volume, dead volume, source volume |
| VOL-003 | Missing inventory volume cannot be assumed. | INSUFFICIENT INFORMATION when available volume is missing. | Available volume field |

# 19. Deterministic Feasibility Logic for App Implementation

System Principle The app should use deterministic rule checks to generate a verdict before any AI explanation is produced. The AI may summarize the deterministic result but should not invent concentration, volume, approval status, plate compatibility, seal type, incubation condition, or missing handling information.

| Input | Rule Engine Check | Output |
| --- | --- | --- |
| Compound ID | Exists in inventory | Pass or insufficient information |
| Inventory Status | Equals Available | Pass or block |
| Expiration Date | Expiration >= request date | Pass, block, or insufficient information |
| Freeze-Thaw Count | Count <= 4 | Pass, block, or insufficient information |
| Available Volume | Available - 10 uL residual >= required | Pass or block |
| Solvent | Apply solvent-specific rule set | Pass, warning, block |
| Plate Type | Compatible with storage, seal, and heat exposure request | Pass, warning, block, or insufficient information |
| Seal Type | Compatible with solvent and plate type | Pass, warning, block |
| Storage / Incubation | Temperature and duration documented and appropriate | Pass, warning, block, or insufficient information |
| Transport | Seal, temperature, and chain-of-custody documented | Pass, warning, block, or insufficient information |

# 20. Scientist-Facing Output Template

The CompoundOps Assistant should present feasibility outputs in a concise, evidence-based format:

Compound ID and solvent

Inventory status and location

Freeze-thaw count and rule result

Available, residual, usable, and required volume

Expiration result

Solvent-specific warnings or blocks

Plate type, seal type, storage, heat exposure, incubation, or transport condition when applicable

Final verdict: FEASIBLE, FEASIBLE WITH WARNING, NOT FEASIBLE, or INSUFFICIENT INFORMATION

Recommended corrective action

# 21. Example Feasibility Output

| Field | Example Output |
| --- | --- |
| Compound ID | CMPD-2051 |
| Solvent | DMSO |
| Plate Type | 384 source plate |
| Seal Type | Solvent-resistant foil seal |
| Storage Condition | -20C frozen storage requested |
| Inventory Status | Available |
| Freeze-Thaw | 8 cycles |
| Available Volume | 13 uL |
| Usable Volume | 3 uL after 10 uL residual |
| SOP Results | FT-001 BLOCK; VOL-001 BLOCK |
| Final Verdict | NOT FEASIBLE |
| Recommendation | Locate alternate vial or generate new stock solution before assay planning. Do not prepare a plate until a feasible source is identified. |

# 22. Source Notes

This training SOP was drafted for the CompoundOps Assistant MVP using the uploaded internal mock SOPs and general solvent-handling reference information gathered during the project discussion. It is intended for educational and prototype demonstration use only, not as a regulated laboratory procedure.

| Source Area | Used For |
| --- | --- |
| DMSO Limits and Solvent Handling SOP | DMSO limits, vehicle controls, residual-volume mention, and AI guardrail framing |
| Freeze-Thaw Control SOP | Freeze-thaw block/pass logic and missing-count handling |
| Compound Storage and Dispensing SOP | Status, expiration, location, residual-volume, and feasibility logic |
| General laboratory solvent safety references | Volatility, flammability, and compatibility cautions for methanol, ethanol, and acetonitrile |
| PBS/buffer reference information | PBS clarity, storage, pH, and compatibility considerations |
| User-provided plate lifecycle requirements | Plate sealing, heat-seal review, storage, incubation, and transport sections |